LiteX Lab Exercises

From Combinatorial Logic to RISC-V SoC

Target: Olimex GateMate A1 EVB

BsC Electronics & Computer Science

Table of Contents

(Update this field in your document editor: right-click and select 'Update Field')

Introduction

These lab exercises introduce FPGA design using the LiteX framework and Migen HDL. LiteX is a Python-based System-on-Chip (SoC) builder that generates Verilog from Python classes, enabling rapid prototyping of FPGA designs without writing VHDL or Verilog by hand.

The five labs form a progressive learning path, starting with simple combinatorial logic and building up to a complete RISC-V SoC with custom peripherals. Each lab is self-contained and can be built and tested on the Olimex GateMate A1 EVB.

Prerequisites

Python 3.8+ with LiteX and Migen installed

Yosys 0.63+ (synthesis tool)

Cologne Chip p_r (place and route for GateMate)

openFPGALoader (bitstream programmer)

Olimex GateMate A1 EVB connected via USB (DirtyJTAG)

Basic knowledge of digital logic (gates, flip-flops, clocks)

Basic Python programming experience

Hardware Setup

The Olimex GateMate A1 EVB uses the Cologne Chip CCGM1A1 FPGA with a 10 MHz on-board oscillator. It has one user LED (IO_SB_B6, active-low) and one user button (IO_SB_B7, active-low). The board connects to the host PC via USB through a DirtyJTAG interface on the RP2040 co-processor, which provides both JTAG programming and UART serial communication.

Key Migen Concepts

Migen is the hardware description language used inside LiteX. Instead of writing VHDL or Verilog, you describe hardware using Python classes:

Module: base class for all hardware blocks

self.comb += : combinatorial (wire) assignments -- no clock, instant propagation

self.sync += : synchronous (registered) assignments -- updates on clock edge

Signal(width): declares a hardware signal (wire or register) with a given bit width

If(condition, ...): conditional hardware multiplexer (not a software if-statement)

platform.request(name): get a physical pin from the board definition

platform.build(module): synthesize, place & route, generate bitstream

Lab 0: Button to LED -- Combinatorial Logic

Objective

Build and deploy your first FPGA design using LiteX/Migen. Understand combinatorial logic, platform IO definitions, and the complete build/program workflow.

Background

Combinatorial logic describes direct connections between inputs and outputs with no memory or clock. When the input changes, the output changes immediately (within propagation delay). This is the simplest possible FPGA design: a button press directly drives an LED.

In Migen, combinatorial assignments use self.comb +=. The .eq() method creates a hardware connection (not a Python assignment). Both the button and LED on the GateMate A1 EVB are active-low: pressing the button drives the pin to 0, and driving the LED pin to 0 turns it on.

Files

00btn2Led/btn2Led.py -- complete design source

Step-by-Step Instructions

1. Navigate to the 00btn2Led/ directory

2. Open btn2Led.py and examine the three main sections:

a) Platform definition (lines 24-35): The _io list defines the FPGA pin mapping. Each entry maps a logical name (e.g., 'user_led_n') to a physical pin (e.g., 'IO_SB_B6'). The Platform class sets the clock frequency and FPGA device.

Platform definition:

_io = [ ("clk0", 0, Pins("IO_SB_A8")), ("user_led_n", 0, Pins("IO_SB_B6")), ("user_btn_n", 0, Pins("IO_SB_B7")), ] class Platform(CologneChipPlatform): default_clk_name = "clk0" default_clk_period = 1e9 / 10e6 # 10 MHz def __init__(self): CologneChipPlatform.__init__(self, "CCGM1A1", _io, toolchain="colognechip")

b) The Btn2Led module (lines 45-50): This is the entire hardware design -- one line of combinatorial logic. self.comb += led.eq(btn) creates a direct wire from the button input to the LED output.

Hardware module:

class Btn2Led(Module): """Combinational button-to-LED module. Drives LED directly from button signal.""" def __init__(self, led, btn): # Both signals are active-low: pass btn straight through to led self.comb += led.eq(btn)

c) The main() function (lines 56-74): This creates the platform, requests the physical signals, builds the design, and programs the FPGA.

Build and program:

def main(): import os, subprocess # Initialize the platform with pin/IO definitions platform = Platform() # Request named signals from the platform configuration led = platform.request("user_led_n", 0) btn = platform.request("user_btn_n", 0) # Instantiate the top-level module module = Btn2Led(led, btn) # Synthesize and place-and-route via the Cologne Chip toolchain build_dir = os.getcwd() + "/build" platform.build(module, build_dir, run=True) # Program the bitstream onto the FPGA subprocess.run(["openFPGALoader", "--cable", "dirtyJtag", "build/top_00.cfg"], check=True)

1. Build and program the FPGA: python btn2Led.py

2. Press the user button on the board and verify the LED turns on

3. Release the button and verify the LED turns off

Full Source Code

btn2Led.py:

""" Button → LED — minimal LiteX/Migen design for the GateMate A1 EVB. Pressing the button drives the LED directly (combinational, active-low). Build: python btn2Led.py Program: openFPGALoader --cable dirtyJtag build/top_00.cfg Note: The _io list and Platform class below can be replaced by importing the upstream board definition: from litex_boards.platforms import olimex_gatemate_a1_evb """ from migen import * from litex.build.generic_platform import Pins, Misc, Subsignal from litex.build.colognechip.platform import CologneChipPlatform # --------------------------------------------------------------------- # Minimal GateMate A1 EVB platform definition # (superseded by litex_boards.platforms.olimex_gatemate_a1_evb) # --------------------------------------------------------------------- _io = [ ("clk0", 0, Pins("IO_SB_A8")), ("user_led_n", 0, Pins("IO_SB_B6")), ("user_btn_n", 0, Pins("IO_SB_B7")), ] class Platform(CologneChipPlatform): default_clk_name = "clk0" default_clk_period = 1e9 / 10e6 # 10 MHz def __init__(self): CologneChipPlatform.__init__(self, "CCGM1A1", _io, toolchain="colognechip") # Create: #+----------------------------------+ #¦ Btn2Led (combinational module) ¦ #¦ - btn_n -> led_n (direct) ¦ #¦ - button pressed → LED on ¦ #¦ - button released → LED off ¦ #+----------------------------------+ class Btn2Led(Module): """Combinational button-to-LED module. Drives LED directly from button signal.""" def __init__(self, led, btn): # Both signals are active-low: pass btn straight through to led self.comb += led.eq(btn) # ------------------ # Build The System # ------------------ def main(): import os, subprocess # Initialize the platform with pin/IO definitions platform = Platform() # Request named signals from the platform configuration led = platform.request("user_led_n", 0) btn = platform.request("user_btn_n", 0) # Instantiate the top-level module module = Btn2Led(led, btn) # Synthesize and place-and-route via the Cologne Chip toolchain build_dir = os.getcwd() + "/build" platform.build(module, build_dir, run=True) # Program the bitstream onto the FPGA subprocess.run(["openFPGALoader", "--cable", "dirtyJtag", "build/top_00.cfg"], check=True) if __name__ == "__main__": main()

Exercises

[Easy] Invert the logic so the LED is ON when the button is NOT pressed. What single change is needed in the Btn2Led class?

[Medium] Add a second LED on a free IO pin (consult the GateMate A1 schematic for available pins). Make it show the inverse of the first LED. You will need to modify both the _io list and the Btn2Led class.

[Hard] Replace the inline platform definition with the upstream board package: 'from litex_boards.platforms import olimex_gatemate_a1_evb'. Update main() to use olimex_gatemate_a1_evb.Platform() and verify the design still works. Compare the upstream platform file to understand what additional resources are defined.

Questions

Q1: What does self.comb += mean in Migen? How is it different from a Python variable assignment?

Q2: Why are both the LED and button 'active-low'? What does led.eq(btn) produce when the button is pressed (pin = 0)?

Q3: What files does platform.build() produce in the build/ directory? What is the .cfg file?

Q4: Why does this design not need a clock signal?

Deliverables

Working bitstream that maps button to LED

Screenshot or photo of build output

Answers to questions Q1-Q4

Modified code for at least one exercise

Lab 1: LED Blink -- Synchronous Logic

Objective

Understand clocked (synchronous) logic in Migen. Use Signal, self.sync, and frequency division through counter bit selection.

Background

Synchronous logic uses a clock signal to coordinate state changes. On every rising edge of the clock, all self.sync assignments are evaluated simultaneously and their results are stored in flip-flops. This is fundamental to almost all practical digital designs.

A free-running counter increments by 1 on every clock cycle. Each bit of the counter toggles at half the frequency of the bit below it. By selecting bit N, you get a signal that toggles every 2^N clock cycles. With a 10 MHz clock and bit 23, the toggle period is 2^23 / 10,000,000 = ~0.84 seconds.

Files

01ledBlink/blinkLed.py -- complete design source

Step-by-Step Instructions

1. Navigate to the 01ledBlink/ directory

2. Open blinkLed.py and note the import: this example uses the upstream platform from litex_boards instead of an inline _io definition

3. Examine the Blink class (lines 21-31):

Blink module:

class Blink(Module): """LED blink module. Bit 23 of a free-running counter drives the LED.""" def __init__(self, led): counter = Signal(26) # Combinatorial assignment: wire counter bit 23 directly to the LED pin self.comb += led.eq(counter[23]) # Synchronous assignment: increment counter every clock cycle self.sync += counter.eq(counter + 1)

Key observations:

Signal(26) declares a 26-bit hardware register (counter)

self.comb += led.eq(counter[23]) wires bit 23 directly to the LED (combinatorial)

self.sync += counter.eq(counter + 1) increments the counter every clock cycle (synchronous)

The counter wraps around automatically when it overflows (26-bit unsigned)

1. Examine the main() function and note the use of platform.create_programmer()

2. Build: python blinkLed.py

3. Observe the LED blinking at approximately 0.6 Hz

Full Source Code

blinkLed.py:

""" LED blink — Migen module that blinks an LED using a free-running counter. Counter bit 23 drives the LED, producing a visible blink rate at 10 MHz clock (2^23 ≈ 8.4 M cycles → ~0.84 s half-period). Build: python blinkLed.py """ from migen import * from litex_boards.targets.olimex_gatemate_a1_evb import * # Create: #+-----------------------------+ #¦ Blink (Module) ¦ #¦ - 26-bit free counter ¦ #¦ - bit 23 → LED ¦ #+-----------------------------+ class Blink(Module): """LED blink module. Bit 23 of a free-running counter drives the LED.""" def __init__(self, led): counter = Signal(26) # Combinatorial assignment: wire counter bit 23 directly to the LED pin self.comb += led.eq(counter[23]) # Synchronous assignment: increment counter every clock cycle self.sync += counter.eq(counter + 1) # ------------------ # Build The System # ------------------ def main(): import os # Initialize the platform with pin/IO definitions platform = olimex_gatemate_a1_evb.Platform() # Get LED signal from platform led = platform.request("user_led_n", 0) # Instantiate top-level module module = Blink(led) # get the build directory build_dir = os.getcwd() + "/build" # Synthesize and place-and-route via the Cologne Chip toolchain platform.build(module, build_dir) # program the chip platform.create_programmer().load_bitstream(build_dir + "/top_00.cfg") if __name__ == "__main__": main()

Exercises

[Easy] Change the blink rate to approximately twice as fast. Which bit index do you use instead of 23? Calculate the exact blink frequency.

[Medium] Add a second output pin that blinks at a different rate (e.g., use bit 20 for one LED and bit 23 for another). You will need to add a pin to the platform or use a PMOD connector.

[Hard] Implement a PWM (pulse-width modulation) dimmer. Use the lower bits of the counter (e.g., counter[0:8]) as a ramp and compare it against a threshold to create a variable duty cycle. The LED should appear to fade or glow at reduced brightness. Hint: self.comb += led.eq(counter[0:8] < threshold)

Questions

Q1: What is the difference between self.comb += and self.sync += in Migen?

Q2: If the clock is 10 MHz and you use bit 23, what is the exact blink period (full cycle, on + off)? Show your calculation.

Q3: Why does this design need a clock signal but Lab 0 did not?

Q4: What happens if you use Signal(8) instead of Signal(26)? Would you still see a visible blink? Why or why not?

Deliverables

Working LED blink at the original rate

Modified blink at a different rate (Exercise 1)

Frequency calculation showing your work

Answers to questions Q1-Q4

Lab 2: Wishbone Bus LED Control -- Bus Protocols & Host Communication

Objective

Understand bus-based peripheral communication using the Wishbone protocol. Compare manual address decoding with the LiteX CSR framework. Control FPGA hardware from a host PC using Python.

Background

The Wishbone bus is a standard interconnect protocol used in FPGA designs. A bus master (e.g., a CPU or UART bridge) initiates transactions to bus slaves (peripherals). The key Wishbone signals are:

CYC (Cycle): indicates a valid bus cycle is in progress

STB (Strobe): master is presenting valid address and data

ACK (Acknowledge): slave confirms it has accepted/provided data

WE (Write Enable): 1 = write transaction, 0 = read transaction

ADR: address bus (word-addressed in LiteX: byte_addr >> 2)

DAT_W / DAT_R: write and read data buses (32-bit)

LiteX provides two approaches for building Wishbone peripherals:

Direct Wishbone slave: you implement the bus interface manually, handling CYC/STB/ACK handshake and address decoding yourself (~50 lines of Migen code)

CSR (Control/Status Register) framework: you declare registers using AutoCSR and CSRStorage, and LiteX auto-generates all bus wiring (~4 lines of code)

Files

02wishBoneMasterAndPerrial/uartWishBoneDirectMapingLed.py -- direct Wishbone slave

02wishBoneMasterAndPerrial/uartWishBoneCrsLed.py -- CSR-based approach

02wishBoneMasterAndPerrial/wishBoneUartDebugLedPeripheralModule.py -- host control script

02wishBoneMasterAndPerrial/testBenchLedPeripheral.py -- simulation testbench

Part A: Direct Wishbone Slave

Open uartWishBoneDirectMapingLed.py and study the WishboneLed class. This implements a complete Wishbone slave with manual address decoding:

WishboneLed -- direct Wishbone slave:

class WishboneLed(Module): """Wishbone slave with built-in address decoding. Always ACKs every transaction to prevent bus hangs on unmapped addresses. Only updates the LED register for writes to the 0x40000400 address. """ def __init__(self, led): self.bus = wishbone.Interface(data_width=32, adr_width=30) led_reg = Signal() # Output signal from the wishbone addr_match = Signal() # True if 0x40000000 # Wishbone (as used by LiteX) uses WORD addressing, not byte addressing. # convert 0x40000400 byte-addr to word address (0x40000400 >> 2) = 0x10000100 # Address decode: byte addr 0x40000000 = word addr 0x10000000 LED_WORD_ADDR = 0x10000100 self.comb += addr_match.eq(self.bus.adr == LED_WORD_ADDR) # addr_match = True if bus.adr= 0x40000000 # Evaluaton and updation the block on the rising edge of the system clock self.sync += [ # WishBone bus, Default: deassert ACK every clock cycle (active for only one cycle) self.bus.ack.eq(0), # Wait for WishBone bus-ready (CYC and STB both asserted, ACK not yet given) # CYC (Cycle) indicates that a valid bus cycle is in progress. # STB (Strobe) indicates that the master is presenting valid address and data on the bus If(self.bus.cyc & self.bus.stb & ~self.bus.ack, # WishBone bus-ready # Respond that the HW is ready (to the bus master by asserting ACK for one cycle) self.bus.ack.eq(1), # :: Warning :: # Unmatched address → is ACK and so bus would hang not hang if user acces wrong address region # Problem We ACK addresses that does not belog to us (might cause timing issues) # If this is a write transaction (WE asserted) and the bus-address matches 0x40000400 If(self.bus.we & addr_match, # Latch the least-significant bit of the write data bus into the LED register led_reg.eq(self.bus.dat_w[0]), ), ), ] self.comb += [ If(addr_match, self.bus.dat_r.eq(led_reg), ).Else( self.bus.dat_r.eq(0), ), led.eq(~led_reg), # active-low: reg=1 → LED on ]

Key observations:

wishbone.Interface(data_width=32, adr_width=30) declares the bus interface

LED_WORD_ADDR = 0x10000100 is the word address (0x40000400 >> 2)

The slave always ACKs every transaction to prevent bus hangs on unmapped addresses

Only writes to the matching address update the LED register

The ACK is registered (self.sync) to last exactly one clock cycle

The Top module wires the UART bridge to the slave:

Top module -- direct wiring:

class Top(Module): def __init__(self, platform): #Get the user I/O pin numbers from litex_boards.platforms.olimex_gatemate_a1_evb led = platform.request("user_led_n", 0) #Get the UART I/O pins numbers from litex_boards.platforms.olimex_gatemate_a1_evb serial = platform.request("serial") # Create a USRT that is connected to the WishBoneBus # UART Wishbone bridge — host PC becomes bus master self.submodules.bridge = bridge = UARTWishboneBridge( pads=serial, clk_freq=CLK_FREQ, baudrate=BAUDRATE, ) # Connect the module led # LED peripheral — direct connection, address decoding is internal # Create the WishboneLed module led_priph and add it to the submodules self.submodules.led = led_periph = WishboneLed(led) #connect the WishboneLed module to the WishBoneBus self.comb += bridge.wishbone.connect(led_periph.bus)

Part B: CSR-Based Approach

Now open uartWishBoneCrsLed.py and compare the CSR approach. The same LED control is achieved with dramatically less code:

LedPeripheral -- CSR approach (4 lines of logic):

class LedPeripheral(Module, AutoCSR): """CSR-mapped LED peripheral. Bit 0 of the control register drives the LED.""" def __init__(self, led): # 1-bit CSR register, directly accessible from the host via litex_server self.control = CSRStorage(1, name="control", description="LED control (bit 0: 1=on, 0=off)") # Active-low LED: reg=1 → pin driven low → LED on self.comb += led.eq(~self.control.storage)

Top(SoCMini) -- automatic CSR wiring:

class Top(SoCMini): # Set CSR region base address to 0x40000000 mem_map = { **SoCMini.mem_map, "csr": 0x40000000, } # Explicitly assign CSR location slots per peripheral # Address = 0x40000000 + location × 0x400 csr_map = { "ctrl": 0, # 0x40000000 "led": 1, # 0x40000400 } def __init__(self, platform): # Initialize SoCMini with UART Wishbone bridge as the bus master. SoCMini.__init__( self, platform, clk_freq=CLK_FREQ, uart_name="crossover", csr_address_width=14, csr_paging=0x400, ) # UART-to-Wishbone bridge — host PC becomes bus master from litex.soc.cores.uart import UARTWishboneBridge serial = platform.request("serial") self.submodules.bridge = UARTWishboneBridge( pads=serial, clk_freq=CLK_FREQ, baudrate=BAUDRATE, ) self.bus.add_master(name="bridge", master=self.bridge.wishbone) # LED peripheral — auto-registered as CSR peripheral via AutoCSR led_pin = platform.request("user_led_n", 0) self.submodules.led = LedPeripheral(led_pin)

Key differences from the direct approach:

AutoCSR eliminates manual bus wiring and ACK logic

CSRStorage(1) automatically creates a readable/writable register

SoCMini provides the CSR decoder and bus interconnect

mem_map and csr_map define the address layout declaratively

csr_paging=0x400 sets the spacing between peripheral address slots

Part C: Host-Side Control

Both variants are controlled from the host PC using the same procedure:

1. Build and program the bitstream: python uartWishBoneCrsLed.py

2. Start the LiteX UART server: litex_server --uart --uart-port=/dev/ttyACM0

3. Run the host control script: python wishBoneUartDebugLedPeripheralModule.py

Host control script excerpt:

from litex import RemoteClient wb = RemoteClient() wb.open() while(True): addr=0x40000400 print(f" Write 0x01 to address 0x{addr:08x} — turns the LED on - please verify") wb.write(addr, 0x1) # Read back to verify value = wb.read(addr) print(f"LED register = 0x{value:08x} (bit0={'ON' if value & 1 else 'OFF'})") input("Press Enter to continue...") print("Turn led off -- please verify") wb.write(addr, 0x0) # Read back to verify value = wb.read(addr) print(f"LED register = 0x{value:08x} (bit0={'ON' if value & 1 else 'OFF'})") input("Press Enter to continue...")

Part D: Simulation (Optional)

The testBenchLedPeripheral.py file demonstrates how to simulate the CSR peripheral without hardware. Run: python testBenchLedPeripheral.py

The testbench creates a Wishbone master that writes to and reads from the LED register, verifying that the LED state changes correctly. It also tests that accesses to unmapped addresses do not affect the LED.

Exercises

[Easy] Modify the host script to blink the LED 5 times with 1-second intervals using time.sleep(). Print the LED state after each write.

[Medium] Add a second CSR register to LedPeripheral: CSRStorage(8, name='brightness'). The hardware does not need to use it yet -- just prove you can write a value from the host and read it back. What address does the new register get? (Hint: check build/csr.csv)

[Hard] In the direct Wishbone variant (uartWishBoneDirectMapingLed.py), the slave ACKs all addresses. Modify it to only ACK when the address matches the LED region. Test what happens when you access an unmapped address from the host. Why is the 'always ACK' pattern used in practice?

Questions

Q1: Why does WishboneLed use word address 0x10000100 for byte address 0x40000400? Show the conversion.

Q2: What is the role of csr_paging=0x400 in the SoCMini configuration? How does it affect the address layout?

Q3: What does AutoCSR do automatically that the WishboneLed class does manually? List at least three things.

Q4: Why is litex_server needed between the host script and the FPGA? What protocol does it bridge?

Deliverables

Both variants built and tested (LED toggles from host)

Host script modification (Exercise 1)

Screenshot of litex_server + host script interaction

Answers to questions Q1-Q4

Lab 3: CRC32 Hardware Accelerator -- HDL Integration & Testing

Objective

Integrate external HDL (Verilog/VHDL) as a black-box peripheral in a LiteX design. Understand Instance(), CSR register types (CSR vs CSRStorage), and simulation-based verification.

Background

CRC32 (Cyclic Redundancy Check) is a checksum algorithm used in Ethernet, ZIP files, and many protocols. The CRC-32/ISO-HDLC variant uses polynomial 0xEDB88320 with initial value 0xFFFFFFFF and a final XOR (bit inversion) step.

The CRC32 step function is a pure combinatorial circuit: given the current accumulator state (32 bits) and one input byte (8 bits), it computes the next accumulator state. This function is implemented in Verilog (hdl/crc.v) or VHDL (hdl/crc.vhdl) and was generated using the crcgen tool (bues.ch/h/crcgen).

In Migen, external HDL modules are wrapped using Instance(). This creates a black-box: Migen does not see the internals, it only knows the port names and widths. The HDL source file is included in synthesis via platform.add_source().

This lab also introduces two CSR register types:

CSR(width): provides .r (data written by host), .w (data read by host), .re (write strobe), .we (read strobe)

CSRStorage(width): provides .storage (persistent register value), .re (write strobe). Simpler for persistent state.

Files

03wishBoneCsrHdl/wishBoneCrsCrc32Verilog.py -- main design (Verilog variant)

03wishBoneCsrHdl/hdl/crc.v -- Verilog CRC32 step function

03wishBoneCsrHdl/hdl/crc.vhdl -- VHDL CRC32 step function (optional)

03wishBoneCsrHdl/testBenchCrc32Peripheral.py -- simulation testbench

03wishBoneCsrHdl/tbLib/crcLib.py -- Python CRC32 reference implementation

Step-by-Step Instructions

Step 1: Examine the Verilog CRC32 module (hdl/crc.v)

This is a pure combinatorial module with 32-bit crcIn, 8-bit data input, and 32-bit crcOut output. It computes one step of the CRC32 algorithm -- no clock, no state.

Step 2: Study the CRC32Peripheral class

CRC32Peripheral with Instance():

class CRC32Peripheral(Module, AutoCSR): """CSR-mapped CRC32 peripheral. Two registers within location slot 2: crc32_data @ 0x40000800 (32-bit rw) write : lower 8 bits are fed into the CRC32 accumulator read : 32-bit running CRC32 checksum (final XOR / bit-inverted accumulator) crc32_reset_ctrl @ 0x40000804 (32-bit w) write : any value → resets the accumulator to 0xFFFFFFFF The accumulator initialises to 0xFFFFFFFF on system reset. The CRC32 step is computed by the VHDL entity in hdl/crc.vhdl. """ def __init__(self, platform): # data : CRS Address CRS_MAP + Offset = 0x40000800 + 0 (first registered CRS) # data : CSR write[7:0] = data byte in, read[31:0] = checksum out # if the bus writes data to the address 0x40000800 self.data is automaticaly updated self.data = CSR(32, name="data") # reset_ctrl CRS :Address CRS_MAP + Offset = 0x40000800 + 4 (second registered CRS) # reset_ctrl CSR: writing any value resets the CRC accumulator to 0xFFFFFFFF # if the bus writes data to the address 0x40000804 self.reset_ctrl is automaticaly updated self.reset_ctrl = CSRStorage(32, name="reset_ctrl", reset=0) # Internal signals — reset values applied automatically on system reset crc_in = Signal(32, reset=0xFFFFFFFF) # Accumulated CRC; CRC32 init = 0xFFFFFFFF crc_out = Signal(32) # Combinatorial output from VHDL crc entity out_buf = Signal(32, reset=0xFFFFFFFF) # Registered checksum; reset matches crc_in # Instantiate the VHDL crc entity (hdl/crc.vhdl) platform.add_source(os.path.join(os.path.dirname(__file__), "hdl/crc.v")) # Connect the verilog signals to this self.specials += Instance("crc", i_crcIn = crc_in, i_data = self.data.r[0:8], # lower 8 bits written by host (c.r = bus dat_w) o_crcOut = crc_out, ) # Reset takes priority: writing reset_ctrl restores accumulator to 0xFFFFFFFF. # On each host data write (data.re fires): feed crcOut back as next crcIn (accumulation). self.sync += [ If(self.reset_ctrl.re, # Writing to 0x40000804 triggers "register enable" (write strobe) for reset_ctrl.re crc_in.eq(0xFFFFFFFF), out_buf.eq(0xFFFFFFFF), ).Elif(self.data.re, # Writing to 0x400008040 triggers "register enable" (write strobe) for data.re crc_in.eq(crc_out), out_buf.eq(crc_out), ) ] # Read path: c.w is what the host reads — invert out_buf for CRC32 final XOR step self.comb += self.data.w.eq(~out_buf)

Key observations:

self.data = CSR(32): host writes trigger .re, written data in .r, read data from .w

self.reset_ctrl = CSRStorage(32): writing any value triggers .re for accumulator reset

Instance('crc', i_crcIn=..., o_crcOut=...): wraps the Verilog module. Prefix i_ means input port, o_ means output port

platform.add_source('hdl/crc.v'): includes the Verilog file in synthesis

Accumulator pattern: crc_out feeds back to crc_in on each write strobe (.re)

Final XOR: self.comb += self.data.w.eq(~out_buf) inverts for CRC32 standard

Step 3: Build and test

1. Build: python wishBoneCrsCrc32Verilog.py

2. Start server: litex_server --uart --uart-port=/dev/ttyACM0

3. Run the validation script to compute CRC32 of test data and verify against Python's binascii.crc32()

Step 4: Run the simulation testbench

The testbench (testBenchCrc32Peripheral.py) uses a pure-Migen replacement for the Verilog Instance, since Migen's simulator cannot simulate external HDL. It validates CRC32 computation against the Python reference implementation.

1. Run: python testBenchCrc32Peripheral.py

2. Examine the output: each test case prints expected vs actual CRC32 values

3. Optional: inspect the generated VCD waveform file

Exercises

[Easy] Write a host script that computes CRC32 of the string 'Hello' using the FPGA peripheral, then verify the result against Python's binascii.crc32(b'Hello'). Print both values.

[Medium] Add a third CSR register byte_count (read-only, 16-bit) that counts how many bytes have been fed into the accumulator since the last reset. Use CSRStatus for a read-only register. Verify from the host that the count increments correctly.

[Hard] Write a simulation test that verifies the CRC32 of the 9-byte string '123456789' equals 0xCBF43926 (the CRC-32/ISO-HDLC check value). Feed bytes one at a time and check intermediate values against the Python reference (tbLib/crcLib.py). This test should catch any bit-ordering or accumulation bugs.

Questions

Q1: Why does the testbench use MigenCRC32Step instead of the Verilog Instance()? What limitation does this work around?

Q2: What is the difference between CSR(32) and CSRStorage(32)? When would you use each?

Q3: Why does the CRC32 accumulator initialize to 0xFFFFFFFF, and why is the output inverted (~out_buf)? What CRC standard requires this?

Q4: What does the .re signal on a CSR register indicate? When exactly does it fire?

Deliverables

Working CRC32 peripheral verified from host

Simulation testbench passing

Host script for 'Hello' CRC32 (Exercise 1)

Answers to questions Q1-Q4

Lab 4: Build Your Own SoC -- RISC-V CPU with Custom Peripherals

Objective

Build a complete RISC-V SoC with custom peripherals. Understand BaseSoC, CPU integration, BIOS firmware, and reusable peripheral libraries.

Background

LiteX can generate a complete System-on-Chip with a RISC-V CPU, memory, UART, and custom peripherals. The BaseSoC class from litex_boards provides a pre-configured SoC for each supported board, including clock domains, memory controller, and UART.

By extending BaseSoC, you inherit a working computer and only need to add your custom peripherals. The LiteX BIOS (a small firmware) boots automatically and provides a command-line interface over UART for memory inspection, peripheral access, and firmware loading.

Two RISC-V CPU options are available for the GateMate A1:

VexRiscV (minimal variant): 3368 CPEs (16.4%), Fmax 27.98 MHz -- recommended

PicoRV32 (minimal variant): 3694 CPEs (18.0%), Fmax 22.62 MHz -- alternative

Prerequisites

All previous labs completed

RISC-V GCC toolchain installed (riscv64-unknown-elf-gcc or similar)

Serial terminal program (picocom, minicom, or screen)

Files

04cpuAndIO/vexriscvLedPeripheral.py -- VexRiscV SoC with LED peripheral

04cpuAndIO/picorvLedAndCrc32Peripherial.py -- PicoRV32 SoC with LED + CRC32

04cpuAndIO/mycsrlib/crc32peripheral.py -- reusable CRC32 peripheral library

04cpuAndIO/programChipOnly.py -- flash pre-built bitstream without rebuild

Step-by-Step Instructions

Step 1: Study the VexRiscV SoC design

vexriscvLedPeripheral.py -- complete source:

#!/usr/bin/env python3 from migen import * from litex_boards.targets.olimex_gatemate_a1_evb import BaseSoC from litex.soc.integration.builder import Builder from litex.soc.integration.soc_core import soc_core_args, soc_core_argdict from litex.soc.interconnect.csr import AutoCSR, CSRStorage import argparse # ------------------------------------------------- # AutoCSR LED Peripheral # ------------------------------------------------- class LedPeripheral(Module, AutoCSR): def __init__(self, led): self.control = CSRStorage(1, name="control", description="LED control (bit 0: 1=on, 0=off)") self.comb += led.eq(~self.control.storage) # ------------------------------------------------- # Custom SoC # ------------------------------------------------- class MySoC(BaseSoC): def __init__(self, **kwargs): kwargs.setdefault("cpu_type", "vexriscv") kwargs.setdefault("cpu_variant", "minimal") kwargs.setdefault("uart_baudrate", 115200) BaseSoC.__init__(self, with_led_chaser=False, # Disable chaser so we can claim user_led_n ourselves **kwargs ) platform = self.platform # Request physical LED from platform (active-low: _n suffix) led = platform.request("user_led_n", 0) # Instantiate peripheral self.submodules.led_peripheral = LedPeripheral(led) # Register as CSR self.add_csr("led_peripheral") # ------------------------------------------------- # Build & Flash # ------------------------------------------------- def main(): parser = argparse.ArgumentParser() soc_core_args(parser) args = parser.parse_args() soc = MySoC(**soc_core_argdict(args)) builder = Builder(soc, output_dir="build", compile_gateware=True, compile_software=True) builder.build() # Flash bitstream to FPGA SRAM via dirtyJtag prog = soc.platform.create_programmer() prog.load_bitstream(builder.get_bitstream_filename(mode="sram")) if __name__ == "__main__": main()

Key observations:

MySoC(BaseSoC): inherits CPU, memory, UART, clock -- everything needed for a computer

kwargs.setdefault('cpu_type', 'vexriscv'): selects the VexRiscV CPU

kwargs.setdefault('cpu_variant', 'minimal'): smallest variant to fit GateMate

with_led_chaser=False: prevents BaseSoC from claiming the LED (so we can use it)

self.add_csr('led_peripheral'): registers our peripheral in the CSR address space

soc_core_args/soc_core_argdict: passes command-line arguments to the SoC builder

compile_software=True: builds the BIOS firmware using the RISC-V toolchain

Step 2: Build the SoC

1. Build: python vexriscvLedPeripheral.py (this takes several minutes -- it compiles BIOS)

2. The build produces a bitstream AND firmware binary

Step 3: Connect and interact with BIOS

1. Program the FPGA (done automatically by the script)

2. Connect a serial terminal: picocom /dev/ttyACM0 -b 115200

3. You should see the LiteX BIOS boot message with system information

4. Try BIOS commands: help, ident, mem_read, mem_write

Step 4: Control the LED from BIOS

Find the LED peripheral CSR address in the build output file build/csr.csv. Use the BIOS mem_write command to toggle the LED:

BIOS commands:

# Example (address may vary -- check csr.csv) mem_write <led_address> 1 # LED on mem_write <led_address> 0 # LED off

Step 5 (Optional): Study the PicoRV32 variant

The picorvLedAndCrc32Peripherial.py file shows how to add the CRC32 peripheral from a reusable library (mycsrlib/). It also demonstrates using PicoRV32 as an alternative CPU. Note the GHDL toolchain setup for VHDL support.

Step 6 (Optional): Study the reusable peripheral library

The mycsrlib/ directory packages the CRC32 peripheral as an importable Python module. This pattern makes peripherals reusable across projects:

Importing a reusable peripheral:

from mycsrlib.crc32peripheral import CRC32Peripheral # In MySoC.__init__: self.submodules.crc32 = CRC32Peripheral(platform) self.add_csr('crc32')

Exercises

[Easy] Use the BIOS mem_write and mem_read commands to toggle the LED from the RISC-V console. Find the LED peripheral CSR address in build/csr.csv and document it.

[Medium] Write a simple C firmware program that blinks the LED using memory-mapped I/O. Use the CSR address from csr.csv. The LiteX BIOS source in build/software/ provides examples of CSR access. Compile and load it via the BIOS serialboot command.

[Hard] Create your own reusable peripheral as a Python class (e.g., a simple timer that counts clock cycles, or a GPIO port with input/output registers). Package it in mycsrlib/, add it to MySoC, rebuild, and verify you can access it from the BIOS console.

Questions

Q1: What does BaseSoC provide that SoCMini (from Lab 2) does not? List at least four components.

Q2: Why is with_led_chaser=False needed when adding a custom LED peripheral?

Q3: What are soc_core_args and soc_core_argdict used for? What kind of parameters can they override?

Q4: Where does the BIOS find the list of CSR peripherals? (Hint: look at the build output files in build/software/include/)

Q5: Compare VexRiscV and PicoRV32 for the GateMate A1. What are the trade-offs in gate count, maximum frequency, and features?

Deliverables

Working SoC with BIOS boot (screenshot of terminal output)

LED controlled from BIOS console (Exercise 1)

csr.csv showing peripheral addresses

Answers to questions Q1-Q5